LOGISTIC ENTERPRISE NETWORK

Project Report

Enterprise Network Design & Implementation

| Domain | Advanced Computer Networks & Systems Engineering |
| --- | --- |
| Course | Computer Networking |
| Instructor | Sir Muhammad Haseeb |
| Program | BS-Software Engineering |
| Institution | Air University, Multan Campus |
| Semester | Spring 2026 |

Project Team

| Member | Name | Roll Number |
| --- | --- | --- |
| Member 1 | Alishba Razzaq | 2510301 |
| Member 2 | Kashmala Rizwan | 2510279 |
| Member 3 | Fatima tu Zahra | 2510280 |

# 1. Executive Summary

This report presents the comprehensive architecture, deployment, and validation of a secure, scalable multi-branch enterprise network topology designed and simulated within Cisco Packet Tracer. The primary objective was to construct a robust infrastructure bridging a central Headquarters (HQ) environment with two distinct remote branch locations, while provisioning a protected edge interface to a simulated Internet Service Provider (ISP).

The network layer employs VLSM to segment a private 192.168.10.0/24 block into efficient, right-sized broadcast domains. Administrative access control and data isolation are enforced through a multi-VLAN architecture (VLANs 10, 20, 30, 40, and 99) integrated via Router-on-a-Stick (RoaS) inter-VLAN routing. Dynamic IP management is automated through native IOS DHCP scopes for each departmental subnet.

Wide-area connectivity is achieved via OSPF Area 0 dynamic routing over serial DCE/DTE point-to-point links. Enterprise edge security is handled by NAT/PAT on a dedicated Edge Router. Branch 2 integrates a WPA2-AES wireless LAN using a WRT300N in transparent bridge mode. All configurations were validated through ICMP ping tests confirming zero packet loss.

# 2. Network Topology — Cisco Packet Tracer

The topology follows a hierarchical enterprise model spanning three physical sites connected over serial WAN links. The full network was built and simulated in Cisco Packet Tracer as shown below.

Figure 1: Full Logistic Enterprise Network Topology — Cisco Packet Tracer

## 2.1 Topology Layers

- Internet Layer: ISP Router (203.0.113.1) provides the simulated public internet boundary.

- Edge Layer: Edge Router performs NAT/PAT translation, separating internal addressing from public IP space.

- Core Layer: Core Router acts as the inter-VLAN routing engine using Router-on-a-Stick sub-interfaces.

- Distribution Layer: HQ Switches provide VLAN access and 802.1Q trunking.

- Access Layer: Branch routers and local switches connect end devices at each remote site.

## 2.2 Connection Types

- Solid lines — 100BASE-TX Straight-Through Ethernet (switch-to-router, switch-to-end-device)

- Dashed lines — Crossover Ethernet cables (switch-to-switch interconnections)

- Red lightning bolt — Serial DCE/DTE point-to-point WAN links

- Dotted lines — 2.4 GHz IEEE 802.11n wireless links (Branch 2)

# 3. IP Address Plan — VLSM Engineering Matrix

The addressing scheme applies VLSM to efficiently carve the 192.168.10.0/24 block into right-sized subnets for each department VLAN and WAN link.

| Device / Interface | VLAN | Purpose | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- | --- |
| Core Router Gi0/0.10 | 10 | Management (HR) | 192.168.10.17 | 255.255.255.240 | — |
| Core Router Gi0/0.20 | 20 | Finance (IT) | 192.168.10.33 | 255.255.255.240 | — |
| Core Router Gi0/0.30 | 30 | IT Support (Sales) | 192.168.10.1 | 255.255.255.240 | — |
| Core Router Gi0/0.40 | 40 | Marketing (Finance) | 192.168.10.49 | 255.255.255.240 | — |
| Core Router Gi0/0.99 | 99 | Native / Guest | 192.168.10.65 | 255.255.255.240 | — |
| Edge Router Serial0/3/0 | — | Internal Core WAN | 192.168.10.105 | 255.255.255.252 | — |
| Edge Router Gi0/0 | — | Public Internet | 203.0.113.2 | 255.255.255.252 | 203.0.113.1 |
| DNS Server Fa0 | 30 | Name Resolution | 192.168.10.10 | 255.255.255.240 | 192.168.10.1 |
| HTTP Server Fa0 | 30 | Corporate Web Portal | 192.168.10.11 | 255.255.255.240 | 192.168.10.1 |

# 4. Configuration Details

## 4.1 Core Router — Inter-VLAN (RoaS) Configuration

The Core Router's GigabitEthernet0/0 is divided into five logical sub-interfaces using 802.1Q encapsulation, enabling all inter-VLAN routing through one physical link.

- interface GigabitEthernet0/0.10 → encapsulation dot1Q 10 → IP: 192.168.10.17/28

- interface GigabitEthernet0/0.20 → encapsulation dot1Q 20 → IP: 192.168.10.33/28

- interface GigabitEthernet0/0.30 → encapsulation dot1Q 30 → IP: 192.168.10.1/28

- interface GigabitEthernet0/0.40 → encapsulation dot1Q 40 → IP: 192.168.10.49/28

- interface GigabitEthernet0/0.99 → encapsulation dot1Q 99 (native) → IP: 192.168.10.65/28

## 4.2 Core Router — DHCP Pools

Native IOS DHCP scopes are configured on the Core Router to automatically distribute network parameters. Each VLAN has a dedicated pool with gateway and server addresses excluded.

| Pool Name | Network Scope | Leased Range | Default Gateway | DNS Server |
| --- | --- | --- | --- | --- |
| Sales_Pool | 192.168.10.0 /28 | 192.168.10.6 – .14 | 192.168.10.1 | 192.168.10.66 |
| HR_Pool | 192.168.10.16 /28 | 192.168.10.22 – .30 | 192.168.10.17 | 192.168.10.66 |
| IT_Pool | 192.168.10.32 /28 | 192.168.10.38 – .46 | 192.168.10.33 | 192.168.10.66 |
| Finance_Pool | 192.168.10.48 /28 | 192.168.10.54 – .62 | 192.168.10.49 | 192.168.10.66 |

## 4.3 Switch Trunking & VLAN Membership

- Trunk Ports — HQ_Switch1 & HQ_Switch2: Fa0/23–24 → switchport mode trunk

- Access Ports — HQ_Switch1: Fa0/2 → VLAN 10, Fa0/3 → VLAN 20, Fa0/4 → VLAN 30

- HQ_Switch2: Fa0/1–2 → VLAN 30, Fa0/3–4 → VLAN 40

- Server_Switch: Fa0/1–2 → VLAN 30, Fa0/3–5 → VLAN 99 (server farm)

## 4.4 OSPF Dynamic WAN Routing

- All routers: router ospf 1 → network advertisements → log-adjacency-changes

- Edge Router: default-information originate — injects 0.0.0.0/0 into OSPF domain

- Core Router neighbors confirmed FULL: 192.168.10.98 (BDR) and 192.168.10.105

## 4.5 Edge Router — NAT/PAT & Default Route

- Serial0/3/0 — ip nat inside  |  GigabitEthernet0/0 — ip nat outside

- Access-list 1 permits 192.168.10.0/24 as NAT source

- ip nat inside source list 1 interface Gi0/0 overload (PAT)

- ip route 0.0.0.0 0.0.0.0 203.0.113.1 — static default toward ISP

# 5. Network Services

## 5.1 DHCP Service

DHCP is handled natively on the Core Router. Verified: PC3 received 192.168.10.23/28 with correct gateway and DNS; PC7 received 192.168.10.6/28.

## 5.2 DNS Resource Records

| Record Name | Type | Resolved Address |
| --- | --- | --- |
| enterprise.local | A Record | 192.168.10.11 — Enterprise Web Portal |
| mail.enterprise.local | A Record | 192.168.10.12 — Email Services Node |

## 5.3 HTTP Corporate Portal — Screenshot

The corporate web server at 192.168.10.11 hosts the internal intranet dashboard. The page was successfully accessed from PC3 using the DNS name http://enterprise.local as shown below.

Figure 2: Corporate Intranet Portal accessed from PC3 via http://enterprise.local

## 5.4 Email Server

Email_Server is configured with SMTP and POP3 active for the enterprise.local domain. User accounts provisioned in the server farm (VLAN 99 at 192.168.10.12).

# 6. Security Policy — ACLs, NAT/PAT & Wireless

## 6.1 ACL 101 — Finance Zone (Gi0/0.20)

| Seq | Action | Source | Destination | Service | Justification |
| --- | --- | --- | --- | --- | --- |
| 10 | Permit | 192.168.10.32/28 | 192.168.10.11/32 | HTTP/HTTPS | Allow Finance dept to access the Corporate Web Portal |
| 20 | Deny | 192.168.10.32/28 | 192.168.10.48/28 | Any | Block Finance from Marketing VLAN lateral movement |
| 30 | Permit | 192.168.10.32/28 | Any | Any | Allow Finance dept all other outbound traffic |

## 6.2 ACL 102 — Management Plane Firewall (VTY 0–4)

| Seq | Action | Source | Destination | Service | Justification |
| --- | --- | --- | --- | --- | --- |
| 10 | Permit | 192.168.10.16/28 | Any | SSH/Telnet | Only Management VLAN may access CLI of any device |
| 20 | Deny | Any | Any | SSH/Telnet | Block all other networks from remote CLI access |

## 6.3 WPA2-AES Wireless Security — Screenshot

Branch 2 wireless LAN secured with WPA2 Personal mode, AES-128 encryption, passphrase Enterprise@2026. The Wireless Router0 configuration is shown below.

Figure 3: WPA2-AES Wireless Security Configuration — Wireless Router0 (Branch 2)

# 7. Verification Evidence

## 7.1 Routing Table

- C 192.168.10.0/28 — directly connected GigabitEthernet0/0.30

- C 192.168.10.16/28 — directly connected GigabitEthernet0/0.10

- C 192.168.10.104/30 — directly connected Serial0/3/0

- O*E2 0.0.0.0/0 [110/1] via 192.168.10.105 — OSPF default from Edge Router

## 7.2 ICMP Ping Test Summary

| Source Device | Source VLAN | Destination IP | Result | Packet Loss |
| --- | --- | --- | --- | --- |
| PC3 | VLAN 30 — IT | 192.168.10.17 (Core Router) | SUCCESS | 0 / 4 |
| PC3 | VLAN 30 — IT | 192.168.10.11 (HTTP Server) | SUCCESS | 0 / 4 |
| Laptop5 | WLAN (Wireless) | 192.168.10.17 (Core Router) | SUCCESS | 0 / 4 |

## 7.3 Ping Test — Packet Tracer Screenshot

The following screenshot from PC3 in Cisco Packet Tracer shows successful ICMP replies from 192.168.10.67, confirming end-to-end inter-VLAN reachability.

Figure 4: ICMP Ping Test from PC3 — Successful Inter-VLAN Connectivity

## 7.4 Wireless Client Connection — Screenshot

Laptop4 in Branch 2 successfully detected and connected to the Branch2_WiFi SSID with WPA2-PSK security, confirming wireless LAN functionality.

Figure 5: Laptop4 Connected to Branch2_WiFi — WPA2-PSK Verified

## 7.5 Additional Service Verification

- DHCP: PC3 obtained 192.168.10.23; PC7 obtained 192.168.10.6 (correct pools)

- DNS: enterprise.local resolves to 192.168.10.67 — verified via PC3 browser

- HTTP: Corporate portal accessible from all internal VLANs by hostname

- NAT: Internal clients successfully reach simulated internet (ISP Router)

- ACL: Finance-to-Marketing blocked; Management SSH access confirmed

# 8. Challenges & Engineering Solutions

During implementation, three significant technical issues were encountered and resolved through methodical troubleshooting.

## Challenge A — Flapping OSPF Neighbor Adjacency States

Problem: The link between Core and Edge routers repeatedly cycled through INIT, EXSTART, and DOWN states, preventing the OSPF topology database from being populated.

Solution: Cleared config with 'no router ospf 1', reconfigured network boundaries, added 'log-adjacency-changes'. Achieved stable FULL adjacency state.

## Challenge B — Gateway of Last Resort Not Propagating

Problem: Core Router displayed 'Gateway of last resort is not set' — all internet traffic dropped even with OSPF active.

Solution: ISP-facing Gi0/0 was administratively down. Assigned IP (203.0.113.2) and ran 'no shutdown'. 'default-information originate' then propagated 0.0.0.0/0 successfully.

## Challenge C — Wireless Laptops Defaulting to APIPA

Problem: Branch 2 laptops fell back to 169.254.x.x APIPA — DHCP discover blocked by double-NAT from WRT300N Internet port.

Solution: Re-patched uplink cable to internal LAN port (Ethernet 1), converting WRT300N to transparent bridge. DHCP addresses assigned correctly from corporate scope immediately.

# 9. Conclusion

All primary project objectives have been successfully completed and verified. The Logistic Enterprise Network demonstrates a production-realistic enterprise design covering inter-VLAN routing, multi-site WAN connectivity, centralized DHCP/DNS services, ACL-enforced traffic segmentation, NAT/PAT edge translation, and secure wireless deployment — all validated through Cisco Packet Tracer.

## Recommended Enhancements

- High Availability — Deploy HSRP/VRRP gateway redundancy at Core Layer.

- AAA Security — Replace local credentials with TACACS+/RADIUS centralised AAA.

- WAN Redundancy — Replace single serial WAN with dual-homed EtherChannel fibre rings.

- Intrusion Prevention — Deploy Cisco Firepower IDS/IPS inline for real-time threat detection.

- Network Monitoring — Implement SolarWinds/PRTG/Zabbix for 24/7 traffic monitoring.